|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR BATTERY ROOM |

| Area/Facility Title: Battery Room |  |  |  |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  | 1 |  |  | 2 |  | 3 |  |  |  | 4 |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Occupancy Or Use |  |  |  |  | ≤once/year |  |  | ≤once/month |  | ≤once/week |  |  |  | ≤once/day |  | ≤once/shift |
|  |  |  |  |  |  |  |  |  | Severity |  |  |  |  | First Aid Only |  |  | Medical Treatment |  | Lost Time |  |  |  | Partial Disability |  | Death or Permanent Disability |
|  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  | Very Unlikely |  |  | Unlikely |  | Possible |  |  |  | Probable |  | Multiple |
|  |  |  | Before Controls |  |  |  |  |  |  |  | After Initial Controls |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |
| Item or activity | Hazard (s) |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E | Risk* (AxBxC)+E | Initial Controls |  |  | Occupancy A | Severity B | Likelihood C |  | Legal Exposure E | Risk = (AxBxC)+E | Control(s) Added to Reduce Risk |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E |  | Risk* (AxBxC)+E | % Risk Reduction |
| Inspection and maintenance at battery room | Low space |  | 2 | 2 | 2 | 0 | 8 | Proper planning, reorganize |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Electrical shock |  | 2 | 1 | 5 | 0 | 10 | Hand gloves, safety shoes, helmet. insulated tools. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Battery orientation |  | 2 | 1 | 5 | 0 | 10 | Proper planning, reorganize |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Electrolyte |  | 2 | 2 | 5 | 0 | 20 | Hand gloves, safety shoes, apron, goggle, helmet, insulated tools. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Suffocation |  | 5 | 1 | 3 | 0 | 15 | Ventilation fan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Low illumination |  | 5 | 1 | 3 | 0 | 15 | Install light |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  |  |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  |  |  | Acceptable |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  | Intolerable |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR BATTERY ROOM |

| Job Title: Battery Room Inspection and maintenance at battery room |  |  |  | Point Value → Parameter ↓ |  |  |  |  |  | 1 |  |  | 2 |  |  | 3 |  |  |  |  | 4 |  |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Frequency (B) |  |  |  |  |  | ≤once/year |  |  | ≤once/month |  |  | ≤once/week |  |  |  |  | ≤once/day |  |  | ≤once/shift |
|  |  |  |  | Severity (C) |  |  |  |  |  | First Aid Only |  |  | Medical Treatment |  |  | Lost Time |  |  |  |  | Partial Disability |  |  | Death or Permanent Disability |
|  |  |  |  | Likelihood (D) |  |  |  |  |  | Extremely Unlikely |  |  | Unlikely |  |  | Possible |  |  |  |  | Probable |  |  | Multiple |
|  |  |  | Before Additional Controls |  |  |  |  |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |  |  |  |
| Job Step/Task | Hazard | Control(s) | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D | Legal Exposure E |  | Risk* (AxBxCxD)+E | Control(s) Added to Reduce Risk |  |  | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D |  | Legal Exposure E | Risk* (AxBxCxD)+E | % Risk Reduction |
| Specific gravity measurement/ Top up. | Spillage | Apron, hand gloves goggles, safety shoes, insulated tools. | N |  | 2 | 2 | 2 | 3 | 0 |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Electrical shock |  | N |  | 2 | 2 | 2 | 2 | 0 |  | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Short circuit |  | N |  | 2 | 2 | 2 | 2 | 0 |  | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cleaning/ Tightening/ Greasing | Slipping of wrench |  | N |  | 2 | 2 | 2 | 2 | 0 |  | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Short circuit |  | N |  | 2 | 2 | 2 | 2 | 0 |  | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cell voltage measurement | Electrical shock |  | N |  | 2 | 2 | 2 | 2 | 0 |  | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Short circuit |  | N |  | 2 | 2 | 2 | 2 | 0 |  | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspection | Suffocation |  | N |  | 2 | 5 | 1 | 2 | 0 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of electrical appliances | Electrocution or injury | All damaged electrical appliances shall be removed from service, use proper PPE | N |  | 2 | 4 | 2 | 3 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR BATTERY ROOM |

(contd.)

| Use of Metallic Tapes and Other Conductive Equipment | Electrocution or injury |  | Metallic tapes & conductive equipment shall avoid where applicable, use of proper PPE |  | N | 2 | 3 |  | 3 | 3 | 0 | 54 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  | 61 to80 |  |  |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  | Acceptable |  |  |  | Moderate |  |  |  |  |  | Substantial |  |  |  |  |  | Intolerable |  |  |  |

Each hazard should occupy one line in the risk table. That is, the risk from each hazard is to be assessed individually. A single activity must be entered three times in the table since there are three hazards associated with this activity.

Note: For legal exposure, the value should be either 5 for identified legal requirements or 0 for no such requirements could be identified.

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR SWITCH YARD (BUS) |

| Area/Facility Title: Switch yard (Bus) |  |  |  |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  | 1 |  |  | 2 |  | 3 |  |  |  | 4 |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Occupancy Or Use |  |  |  |  | ≤once/year |  |  | ≤once/month |  | ≤once/week |  |  |  | ≤once/day |  | ≤once/shift |
|  |  |  |  |  |  |  |  |  | Severity |  |  |  |  | First Aid Only |  |  | Medical Treatment |  | Lost Time |  |  |  | Partial Disability |  | Death or Permanent Disability |
|  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  | Very Unlikely |  |  | Unlikely |  | Possible |  |  |  | Probable |  | Multiple |
|  |  |  | Before Controls |  |  |  |  |  |  |  | After Initial Controls |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |
| Item or activity | Hazard (s) |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E | Risk* (AxBxC)+E | Initial Controls |  |  | Occupancy A | Severity B | Likelihood C |  | Legal Exposure E | Risk = (AxBxC)+E | Control(s) Added to Reduce Risk |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E |  | Risk* (AxBxC)+E | % Risk Reduction |
| Schedule & Emergency Maintenance | HV shock |  | 3 | 5 | 4 | 0 | 60 | Proper isolation & grounding, hand gloves, safety shoes, safety belt. Helmet. Ladder. Safe distance from live parts. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fall down from height |  | 2 | 5 | 4 | 0 | 40 | Hand gloves, safety shoes, safety belt. Helmet. Ladder |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overhead Lines | Contact with or near approach to a live high voltage |  | 5 | 4 | 4 | 0 | 80 | Use proper PPE, maintain safe distance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  |  |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  |  |  | Acceptable |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  | Intolerable |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR SWITCH YARD (BUS) |

| Job Title: Switch yard (Bus) Schedule & Emergency Maintenance |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  |  |  | 1 |  |  | 2 |  |  |  | 3 |  |  |  |  | 4 |  |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Frequency (B) |  |  |  |  |  |  | ≤once/year |  |  | ≤once/month |  |  |  | ≤once/week |  |  |  |  | ≤once/day |  |  | ≤once/shift |
|  |  |  |  |  |  | Severity (C) |  |  |  |  |  |  | First Aid Only |  |  | Medical Treatment |  |  |  | Lost Time |  |  |  |  | Partial Disability |  |  | Death or Permanent Disability |
|  |  |  |  |  |  | Likelihood (D) |  |  |  |  |  |  | Extremely Unlikely |  |  | Unlikely |  |  |  | Possible |  |  |  |  | Probable |  |  | Multiple |
|  |  |  |  |  | Before Additional Controls |  |  |  |  |  |  |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |  |  |  |
| Job Step/Task | Hazard |  | Control(s) |  | Stressors Y/N |  | # of People A | Frequency B |  | Severity C | Likelihood D | Legal Exposure E |  | Risk* (AxBxCxD)+E | Control(s) Added to Reduce Risk |  |  |  | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D |  | Legal Exposure E | Risk* (AxBxCxD)+E | % Risk Reduction |
| Cleaning/ Tightening/Nut Bolt changing, | Stressors |  | Proper isolation & grounding, hand gloves, safety shoes, safety belt, helmet. Proper use of ladder, |  | Y |  | 2 | 2 |  | 3 | 3 | 0 |  | 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fall down From height |  |  |  | Y |  | 2 | 1 |  | 5 | 4 | 0 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Changing Disc insulator. | Stressors |  |  |  | Y |  | 2 | 2 |  | 2 | 3 | 0 |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fall down From height |  |  |  | N |  | 2 | 2 |  | 3 | 4 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of electrical appliances | Electrocution or injury |  | All damaged electrical appliances shall be removed from service, use proper PPE |  | N |  | 2 | 4 |  | 2 | 3 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of Metallic Tapes and Other Conductive Equipment | Electrocution or injury |  | Metallic tapes & conductive equipment shall avoid where applicable, use of proper PPE |  | N |  | 2 | 3 |  | 3 | 3 | 0 |  | 54 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  | 21 to40 |  |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  |  |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  | Acceptable |  |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  |  |  |  |  | Intolerable |  |  |  |

Each hazard should occupy one line in the risk table. That is, the risk from each hazard is to be assessed individually. A single activity must be entered three times in the table since there are three hazards associated with this activity.

Note: For legal exposure, the value should be either 5 for identified legal requirements or 0 for no such requirements could be identified.

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR SWITCH YARD (CIRCUIT BREAKER) |

| Area/Facility Title: Switch yard (Circuit Breaker) |  |  |  |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  | 1 |  |  | 2 |  | 3 |  |  |  | 4 |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Occupancy Or Use |  |  |  |  | ≤once/year |  |  | ≤once/month |  | ≤once/week |  |  |  | ≤once/day |  | ≤once/shift |
|  |  |  |  |  |  |  |  |  | Severity |  |  |  |  | First Aid Only |  |  | Medical Treatment |  | Lost Time |  |  |  | Partial Disability |  | Death or Permanent Disability |
|  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  | Very Unlikely |  |  | Unlikely |  | Possible |  |  |  | Probable |  | Multiple |
|  |  |  | Before Controls |  |  |  |  |  |  |  | After Initial Controls |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |
| Item or activity | Hazard (s) |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E | Risk* (AxBxC)+E | Initial Controls |  |  | Occupancy A | Severity B | Likelihood C |  | Legal Exposure E | Risk = (AxBxC)+E | Control(s) Added to Reduce Risk |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E |  | Risk* (AxBxC)+E | % Risk Reduction |
| Schedule & Emergency maintenance, | Height |  | 3 | 5 | 3 | 0 | 45 | Hang gloves, safety belt, safety shoes, helmet, ladder. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | High Voltage shock |  | 3 | 5 | 4 | 0 | 60 | Proper grounding, hang gloves, safety belt, safety shoes, helmet, ladder |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Porcelain Blast |  | 1 | 5 | 5 | 0 | 25 | Safety shoes, helmet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bird or squirrel or snake |  | 4 | 3 | 4 | 0 | 48 | Inspection, horn, air gun, regular cleaning of grasses and bushes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overhead Lines | Contact with or near approach to a live high voltage |  | 5 | 4 | 4 | 0 | 80 | Use proper PPE, maintain safe distance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  |  |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  |  |  | Acceptable |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  | Intolerable |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR SWITCH YARD (CIRCUIT BREAKER) |

| Job Title: Switch yard (Circuit Breaker) Emergency or Schedule maintenance |  |  |  | Point Value → Parameter ↓ |  |  |  |  |  | 1 |  |  | 2 |  |  | 3 |  |  |  |  | 4 |  |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Frequency (B) |  |  |  |  |  | ≤once/year |  |  | ≤once/month |  |  | ≤once/week |  |  |  |  | ≤once/day |  |  | ≤once/shift |
|  |  |  |  | Severity (C) |  |  |  |  |  | First Aid Only |  |  | Medical Treatment |  |  | Lost Time |  |  |  |  | Partial Disability |  |  | Death or Permanent Disability |
|  |  |  |  | Likelihood (D) |  |  |  |  |  | Extremely Unlikely |  |  | Unlikely |  |  | Possible |  |  |  |  | Probable |  |  | Multiple |
|  |  |  | Before Additional Controls |  |  |  |  |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |  |  |  |
| Job Step/Task | Hazard | Control(s) | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D | Legal Exposure E |  | Risk* (AxBxCxD)+E | Control(s) Added to Reduce Risk |  |  | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D |  | Legal Exposure E | Risk* (AxBxCxD)+E | % Risk Reduction |
| Cleaning/ Nut bolt tightening/ CB changing / Pole changing, | Stress | Hang gloves, safety belt, safety shoes, helmet, ladder, proper tools & tackle chain ton. | Y |  | 4 | 2 | 3 | 3 | 0 |  | 72 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fall down from height. |  | Y |  | 2 | 2 | 4 | 4 | 0 |  | 64 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pole or C.B charge |  | Y |  | 5 | 1 | 5 | 2 | 0 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| During grounding led connection. | Voltage induction | Hand gloves safety belt, safety shoes, helmet, hot stick. | Y |  | 1 | 2 | 3 | 3 | 0 |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Measuring Insulation & conductance. | LV shock | Hand gloves safety belt, safety shoes, helmet, grounding, | Y |  | 2 | 1 | 4 | 4 | 0 |  | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of electrical appliances | Electrocution or injury | All damaged electrical appliances shall be removed from service, use proper PPE | N |  | 2 | 4 | 2 | 3 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR SWITCH YARD (CIRCUIT BREAKER) |

(Contd.)

| Use of Metallic Tapes and Other Conductive Equipment | Electrocution or injury |  | Metallic tapes & conductive equipment shall avoid where applicable, use of proper PPE |  | N | 2 | 3 |  | 3 | 3 | 0 | 54 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  | 61 to80 |  |  |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  | Acceptable |  |  |  | Moderate |  |  |  |  |  | Substantial |  |  |  |  |  | Intolerable |  |  |  |

Each hazard should occupy one line in the risk table. That is, the risk from each hazard is to be assessed individually. A single activity must be entered three times in the table since there are three hazards associated with this activity.

Note: For legal exposure, the value should be either 5 for identified legal requirements or 0 for no such requirements could be identified.

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR SWITCH YARD (CURRENT TRANSFORMER) |

| Area/Facility Title: Switch yard (Current Transformer) |  |  |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  | 1 |  |  | 2 | 3 |  |  |  | 4 |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Occupancy Or Use |  |  |  | ≤once/year |  |  | ≤once/month | ≤once/week |  |  |  | ≤once/day |  | ≤once/shift |
|  |  |  |  |  |  |  |  | Severity |  |  |  | First Aid Only |  |  | Medical Treatment | Lost Time |  |  |  | Partial Disability |  | Death or Permanent Disability |
|  |  |  |  |  |  |  |  | Likelihood |  |  |  | Very Unlikely |  |  | Unlikely | Possible |  |  |  | Probable |  | Multiple |
|  |  | Before Controls |  |  |  |  |  |  | After Initial Controls |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |
| Item or activity | Hazard (s) | Occupancy A | Severity B | Likelihood C | Legal Exposure E | Risk* (AxBxC)+E | Initial Controls |  | Occupancy A | Severity B | Likelihood C |  | Legal Exposure E | Risk = (AxBxC)+E | Control(s) Added to Reduce Risk | Occupancy A | Severity B | Likelihood C | Legal Exposure E |  | Risk* (AxBxC)+E | % Risk Reduction |
| Schedule & Emergency maintenance. | Fall from height | 3 | 5 | 3 | 0 | 45 | Hang gloves, safety belt, safety shoes, helmet, proper use of ladder. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | High Voltage shock | 3 | 5 | 4 | 0 | 60 | Proper Grounding, hang gloves, Safety belt, safety shoes, helmet, proper use of ladder |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Schedule & Emergency maintenance | Porcelain Blast | 1 | 5 | 5 | 0 | 25 | safety shoes, helmet. Maintaining safe distance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bird or squirrel or snake | 4 | 2 | 4 | 0 | 32 | Inspection, horn, air Gun, regular cleaning of grasses and bushes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR SWITCH YARD (CURRENT TRANSFORMER) |

(Contd.)

| Overhead Lines | Contact with or near approach to a live high voltage |  | 5 | 4 | 4 | 0 | 80 | Use proper PPE, maintain safe distance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  |  |  | 21 to40 |  |  | 41 to60 |  |  |  |  |  |  | 61 to80 |  |  |  | 81 or greater |  |  |
|  |  | Negligible |  |  |  | Acceptable |  |  | Moderate |  |  |  |  |  |  | Substantial |  |  |  | Intolerable |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR SWITCH YARD (CURRENT TRANSFORMER) |

| Job Title: Switch yard (Current Transformer) Emergency and Schedule maintenance |  |  |  | Point Value → Parameter ↓ |  |  |  |  |  | 1 |  |  | 2 |  |  | 3 |  |  |  |  | 4 |  |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Frequency (B) |  |  |  |  |  | ≤once/year |  |  | ≤once/month |  |  | ≤once/week |  |  |  |  | ≤once/day |  |  | ≤once/shift |
|  |  |  |  | Severity (C) |  |  |  |  |  | First Aid Only |  |  | Medical Treatment |  |  | Lost Time |  |  |  |  | Partial Disability |  |  | Death or Permanent Disability |
|  |  |  |  | Likelihood (D) |  |  |  |  |  | Extremely Unlikely |  |  | Unlikely |  |  | Possible |  |  |  |  | Probable |  |  | Multiple |
|  |  |  | Before Additional Controls |  |  |  |  |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |  |  |  |
| Job Step/Task | Hazard | Control(s) | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D | Legal Exposure E |  | Risk* (AxBxCxD)+E | Control(s) Added to Reduce Risk |  |  | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D |  | Legal Exposure E | Risk* (AxBxCxD)+E | % Risk Reduction |
| Cleaning/ Nut bolt tightening/C.T changing | Stress | Hang gloves, safety belt, safety shoes, helmet, ladder, proper tools & tackle chain ton. | Y |  | 4 | 2 | 3 | 3 | 0 |  | 72 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fall down from height. |  | Y |  | 2 | 1 | 4 | 4 | 0 |  | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | C.T change |  | Y |  | 5 | 1 | 5 | 2 | 0 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| During grounding led connection. | Voltage induction | Hand gloves safety belt, safety shoes, helmet, Hot stick. | Y |  | 1 | 2 | 2 | 3 | 0 |  | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Measuring Insulation & conductance. | LV shock | Hand gloves safety belt, safety shoes, helmet, grounding, | Y |  | 2 | 1 | 4 | 3 | 0 |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of electrical appliances | Electrocution or injury | All damaged electrical appliances shall be removed from service, use proper PPE | N |  | 2 | 4 | 2 | 3 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR SWITCH YARD (CURRENT TRANSFORMER) |

(Contd.)

| Use of Metallic Tapes and Other Conductive Equipment | Electrocution or injury |  | Metallic tapes & conductive equipment shall avoid where applicable, use of proper PPE |  | N | 2 | 3 |  | 3 | 3 | 0 | 54 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  | 61 to80 |  |  |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  | Acceptable |  |  |  | Moderate |  |  |  |  |  | Substantial |  |  |  |  |  | Intolerable |  |  |  |

Each hazard should occupy one line in the risk table. That is, the risk from each hazard is to be assessed individually. A single activity must be entered three times in the table since there are three hazards associated with this activity.

Note: For legal exposure, the value should be either 5 for identified legal requirements or 0 for no such requirements could be identified.

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR CAPACITOR BANK |

| Area/Facility Title: Capacitor Bank |  |  |  |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  | 1 |  |  | 2 |  | 3 |  |  |  | 4 |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Occupancy Or Use |  |  |  |  | ≤once/year |  |  | ≤once/month |  | ≤once/week |  |  |  | ≤once/day |  | ≤once/shift |
|  |  |  |  |  |  |  |  |  | Severity |  |  |  |  | First Aid Only |  |  | Medical Treatment |  | Lost Time |  |  |  | Partial Disability |  | Death or Permanent Disability |
|  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  | Very Unlikely |  |  | Unlikely |  | Possible |  |  |  | Probable |  | Multiple |
|  |  |  | Before Controls |  |  |  |  |  |  |  | After Initial Controls |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |
| Item or activity | Hazard (s) |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E | Risk* (AxBxC)+E | Initial Controls |  |  | Occupancy A | Severity B | Likelihood C |  | Legal Exposure E | Risk = (AxBxC)+E | Control(s) Added to Reduce Risk |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E |  | Risk* (AxBxC)+E | % Risk Reduction |
| Emergency & schedule maintenance | Low Space |  | 1 | 5 | 3 | 0 | 15 | Proper use of PPE, regular cleaning |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | High Voltage shock |  | 1 | 5 | 4 | 0 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Capacitor Blast |  | 1 | 5 | 5 | 0 | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bird or squirrel |  | 3 | 2 | 3 | 0 | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overhead Lines | Contact with or near approach to a live high voltage |  | 5 | 4 | 4 | 0 | 80 | Use proper PPE, maintain safe distance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  |  |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  |  |  | Acceptable |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  | Intolerable |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR CAPACITOR BANK |

| Job Title: Capacitor Bank Emergency and Schedule maintenance |  |  |  | Point Value → Parameter ↓ |  |  |  |  |  | 1 |  |  | 2 |  |  | 3 |  |  |  |  | 4 |  |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Frequency (B) |  |  |  |  |  | ≤once/year |  |  | ≤once/month |  |  | ≤once/week |  |  |  |  | ≤once/day |  |  | ≤once/shift |
|  |  |  |  | Severity (C) |  |  |  |  |  | First Aid Only |  |  | Medical Treatment |  |  | Lost Time |  |  |  |  | Partial Disability |  |  | Death or Permanent Disability |
|  |  |  |  | Likelihood (D) |  |  |  |  |  | Extremely Unlikely |  |  | Unlikely |  |  | Possible |  |  |  |  | Probable |  |  | Multiple |
|  |  |  | Before Additional Controls |  |  |  |  |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |  |  |  |
| Job Step/Task | Hazard | Control(s) | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D | Legal Exposure E |  | Risk* (AxBxCxD)+E | Control(s) Added to Reduce Risk |  |  | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D |  | Legal Exposure E | Risk* (AxBxCxD)+E | % Risk Reduction |
| Cleaning/ Nut bolt tightening/ LA changing/ Cap. cell change, capacitance/ inductance test | NCT charge from semi dead part | Hang gloves, safety belt, safety shoes, helmet, 15 min delay after shutdown. Proper grounding. | Y |  | 3 | 1 | 4 | 3 | 0 |  | 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | HV shock from adjacent live part |  | Y |  | 2 | 1 | 4 | 4 | 0 |  | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Capacitor charge from semi dead part |  | Y |  | 2 | 1 | 5 | 3 | 0 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| During grounding led connection. | Voltage induction | Hand gloves safety belt, safety shoes, helmet | Y |  | 1 | 1 | 4 | 3 | 0 |  | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Measuring capacitance | HV shock or Voltage induction | Hand gloves safety belt, safety shoes, helmet | Y |  | 2 | 1 | 4 | 3 | 0 |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of electrical appliances | Electrocution or injury | All damaged electrical appliances shall be removed from service, use proper PPE | N |  | 2 | 4 | 2 | 3 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR CAPACITOR BANK |

(Contd.)

| Use of Metallic Tapes and Other Conductive Equipment | Electrocution or injury |  | Metallic tapes & conductive equipment shall avoid where applicable, use of proper PPE |  | N | 2 | 3 |  | 3 | 3 | 0 | 54 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  | 61 to80 |  |  |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  | Acceptable |  |  |  | Moderate |  |  |  |  |  | Substantial |  |  |  |  |  | Intolerable |  |  |  |

Each hazard should occupy one line in the risk table. That is, the risk from each hazard is to be assessed individually. A single activity must be entered three times in the table since there are three hazards associated with this activity.

Note: For legal exposure, the value should be either 5 for identified legal requirements or 0 for no such requirements could be identified.

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR CONTROL ROOM |

| Area/Facility Title: Control Room |  |  |  |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  | 1 |  |  | 2 |  | 3 |  |  |  | 4 |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Occupancy Or Use |  |  |  |  | ≤once/year |  |  | ≤once/month |  | ≤once/week |  |  |  | ≤once/day |  | ≤once/shift |
|  |  |  |  |  |  |  |  |  | Severity |  |  |  |  | First Aid Only |  |  | Medical Treatment |  | Lost Time |  |  |  | Partial Disability |  | Death or Permanent Disability |
|  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  | Very Unlikely |  |  | Unlikely |  | Possible |  |  |  | Probable |  | Multiple |
|  |  |  | Before Controls |  |  |  |  |  |  |  | After Initial Controls |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |
| Item or activity | Hazard (s) |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E | Risk* (AxBxC)+E | Initial Controls |  |  | Occupancy A | Severity B | Likelihood C |  | Legal Exposure E | Risk = (AxBxC)+E | Control(s) Added to Reduce Risk |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E |  | Risk* (AxBxC)+E | % Risk Reduction |
| Room space | Low space |  | 2 | 3 | 4 | 0 | 24 | Hand gloves, safety shoes, helmet. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Control panel Inspection. | Shock(AC/DC) |  | 1 | 3 | 5 | 0 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Control panel | Emit heat form device and heater |  | 5 | 2 | 5 | 0 | 50 | Effective functioning of ac |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CT terminal open | Meter burning |  | 1 | 5 | 3 | 0 | 15 | Proper tighten. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Short ckt. | Fire |  | 3 | 5 | 3 | 0 | 45 | Firefighting equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lose connection | Body not earthed properly |  | 5 | 2 | 1 | 0 | 10 | Proper tighten. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  |  |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  |  |  | Acceptable |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  | Intolerable |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR CONTROL ROOM |

| Job Title: Control Room Operation, Control Panel Maintenance, Test, Troubleshooting |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  |  |  | 1 |  |  | 2 |  |  |  | 3 |  |  |  |  | 4 |  |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Frequency (B) |  |  |  |  |  |  | ≤once/year |  |  | ≤once/month |  |  |  | ≤once/week |  |  |  |  | ≤once/day |  |  | ≤once/shift |
|  |  |  |  |  |  | Severity (C) |  |  |  |  |  |  | First Aid Only |  |  | Medical Treatment |  |  |  | Lost Time |  |  |  |  | Partial Disability |  |  | Death or Permanent Disability |
|  |  |  |  |  |  | Likelihood (D) |  |  |  |  |  |  | Extremely Unlikely |  |  | Unlikely |  |  |  | Possible |  |  |  |  | Probable |  |  | Multiple |
|  |  |  |  |  | Before Additional Controls |  |  |  |  |  |  |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |  |  |  |
| Job Step/Task | Hazard |  | Control(s) |  | Stressors Y/N |  | # of People A | Frequency B |  | Severity C | Likelihood D | Legal Exposure E |  | Risk* (AxBxCxD)+E | Control(s) Added to Reduce Risk |  |  |  | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D |  | Legal Exposure E | Risk* (AxBxCxD)+E | % Risk Reduction |
| Operation/Cleaning/ Wiring/ Continuity test/ functionality test/ troubleshooting. | LV AC shock |  | Hand gloves, safety shoes, helmet. Insulated tools. |  | N |  | 2 | 2 |  | 5 | 3 | 0 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 220 or 110 VDC shock |  |  |  | N |  | 2 | 1 |  | 3 | 5 | 0 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Short Ckt. |  |  |  | N |  | 2 | 1 |  | 4 | 2 | 0 |  | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Burning from Electric heater |  |  |  | N |  | 2 | 1 |  | 1 | 2 | 0 |  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of electrical appliances | Electrocution or injury |  | All damaged electrical appliances shall be removed from service, use proper PPE |  | N |  | 2 | 4 |  | 2 | 3 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of Metallic Tapes and Other Conductive Equipment | Electrocution or injury |  | Metallic tapes & conductive equipment shall avoid where applicable, use of proper PPE |  | N |  | 2 | 3 |  | 3 | 3 | 0 |  | 54 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  | 21 to40 |  |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  |  |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  | Acceptable |  |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  |  |  |  |  | Intolerable |  |  |  |

Each hazard should occupy one line in the risk table. That is, the risk from each hazard is to be assessed individually. A single activity must be entered three times in the table since there are three hazards associated with this activity.

Note: For legal exposure, the value should be either 5 for identified legal requirements or 0 for no such requirements could be identified.

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR SWITCH YARD (ISOLATOR) |

| Area/Facility Title: Switch yard (Isolator) |  |  |  |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  | 1 |  |  | 2 |  | 3 |  |  |  | 4 |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Occupancy Or Use |  |  |  |  | ≤once/year |  |  | ≤once/month |  | ≤once/week |  |  |  | ≤once/day |  | ≤once/shift |
|  |  |  |  |  |  |  |  |  | Severity |  |  |  |  | First Aid Only |  |  | Medical Treatment |  | Lost Time |  |  |  | Partial Disability |  | Death or Permanent Disability |
|  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  | Very Unlikely |  |  | Unlikely |  | Possible |  |  |  | Probable |  | Multiple |
|  |  |  | Before Controls |  |  |  |  |  |  |  | After Initial Controls |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |
| Item or activity | Hazard (s) |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E | Risk* (AxBxC)+E | Initial Controls |  |  | Occupancy A | Severity B | Likelihood C |  | Legal Exposure E | Risk = (AxBxC)+E | Control(s) Added to Reduce Risk |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E |  | Risk* (AxBxC)+E | % Risk Reduction |
| Schedule & Emergency maintenance | Fall from height |  | 3 | 5 | 3 | 0 | 45 | Safety belt, safety shoes, helmet, proper use of ladder. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | High Voltage shock |  | 3 | 5 | 3 | 0 | 45 | Proper grounding, hang gloves, safety belt, safety shoes, helmet, ladder |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Porcelain Blast |  | 1 | 5 | 5 | 0 | 25 | Safety shoes, helmet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bird or squirrel or snake |  | 4 | 2 | 4 | 0 | 32 | Inspection, horn, air gun, regular cleaning of grasses & bushes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overhead Lines | Contact with or near approach to a live high voltage |  | 5 | 4 | 4 | 0 | 80 | Use proper PPE, maintain safe distance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  |  |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  |  |  | Acceptable |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  | Intolerable |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR SWITCH YARD (ISOLATOR) |

| Job Title: Switch Yard (Isolator) Emergency or Schedule maintenance |  |  |  | Point Value → Parameter ↓ |  |  |  |  |  | 1 |  |  | 2 |  |  | 3 |  |  |  |  | 4 |  |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Frequency (B) |  |  |  |  |  | ≤once/year |  |  | ≤once/month |  |  | ≤once/week |  |  |  |  | ≤once/day |  |  | ≤once/shift |
|  |  |  |  | Severity (C) |  |  |  |  |  | First Aid Only |  |  | Medical Treatment |  |  | Lost Time |  |  |  |  | Partial Disability |  |  | Death or Permanent Disability |
|  |  |  |  | Likelihood (D) |  |  |  |  |  | Extremely Unlikely |  |  | Unlikely |  |  | Possible |  |  |  |  | Probable |  |  | Multiple |
|  |  |  | Before Additional Controls |  |  |  |  |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |  |  |  |
| Job Step/Task | Hazard | Control(s) | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D | Legal Exposure E |  | Risk* (AxBxCxD)+E | Control(s) Added to Reduce Risk |  |  | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D |  | Legal Exposure E | Risk* (AxBxCxD)+E | % Risk Reduction |
| Cleaning/ Nut bolt tightening / Isolator changing | Stress | Hang gloves, safety belt, safety shoes, helmet, ladder, proper tools & tackle chain ton. | Y |  | 4 | 2 | 3 | 3 | 0 |  | 72 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fall down from height. |  | Y |  | 2 | 1 | 4 | 4 | 0 |  | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | P.T charge |  | Y |  | 5 | 1 | 5 | 2 | 0 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| During grounding led connection. | Voltage induction | Hand gloves safety belt, safety shoes, helmet, hot stick. | Y |  | 1 | 2 | 3 | 3 | 0 |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Measuring Insulation, Contact resistance. | LV shock | Hand gloves safety belt, safety shoes, helmet, grounding, | Y |  | 2 | 1 | 4 | 3 | 0 |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of electrical appliances | Electrocution or injury | All damaged electrical appliances shall be removed from service, use proper PPE | N |  | 2 | 4 | 2 | 3 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR SWITCH YARD (ISOLATOR) |

(Contd.)

| Use of Metallic Tapes and Other Conductive Equipment | Electrocution or injury |  | Metallic tapes & conductive equipment shall avoid where applicable, use of proper PPE |  | N | 2 | 3 |  | 3 | 3 | 0 | 54 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  | 61 to80 |  |  |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  | Acceptable |  |  |  | Moderate |  |  |  |  |  | Substantial |  |  |  |  |  | Intolerable |  |  |  |

Each hazard should occupy one line in the risk table. That is, the risk from each hazard is to be assessed individually. A single activity must be entered three times in the table since there are three hazards associated with this activity.

Note: For legal exposure, the value should be either 5 for identified legal requirements or 0 for no such requirements could be identified.

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR LIGHTENING ARRESTOR |

| Area/Facility Title: Switch yard (Lightning Arrestor) |  |  |  |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  | 1 |  |  | 2 |  | 3 |  |  |  | 4 |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Occupancy Or Use |  |  |  |  | ≤once/year |  |  | ≤once/month |  | ≤once/week |  |  |  | ≤once/day |  | ≤once/shift |
|  |  |  |  |  |  |  |  |  | Severity |  |  |  |  | First Aid Only |  |  | Medical Treatment |  | Lost Time |  |  |  | Partial Disability |  | Death or Permanent Disability |
|  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  | Very Unlikely |  |  | Unlikely |  | Possible |  |  |  | Probable |  | Multiple |
|  |  |  | Before Controls |  |  |  |  |  |  |  | After Initial Controls |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |
| Item or activity | Hazard (s) |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E | Risk* (AxBxC)+E | Initial Controls |  |  | Occupancy A | Severity B | Likelihood C |  | Legal Exposure E | Risk = (AxBxC)+E | Control(s) Added to Reduce Risk |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E |  | Risk* (AxBxC)+E | % Risk Reduction |
| Schedule & Emergency maintenance. | Fall from height |  | 3 | 5 | 4 | 0 | 60 | Safety belt, safety shoes, helmet, ladder. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | High Voltage shock |  | 3 | 5 | 4 | 0 | 60 | Proper grounding, hang gloves, safety belt, safety shoes, helmet, ladder |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Porcelain Blast |  | 1 | 5 | 5 | 0 | 25 | Safety shoes, helmet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bird or squirrel or snake |  | 4 | 2 | 4 | 0 | 32 | Inspection, horn, air gun, regular cleaning of grasses and bushes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overhead Lines | Contact with or near approach to a live high voltage |  | 5 | 4 | 4 | 0 | 80 | Use proper PPE, maintain safe distance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  |  |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  |  |  | Acceptable |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  | Intolerable |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR LIGHTENING ARRESTOR |

| Job Title: Switch yard (Lightning Arrestor) Emergency or Schedule maintenance |  |  |  | Point Value → Parameter ↓ |  |  |  |  |  | 1 |  |  | 2 |  |  | 3 |  |  |  |  | 4 |  |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Frequency (B) |  |  |  |  |  | ≤once/year |  |  | ≤once/month |  |  | ≤once/week |  |  |  |  | ≤once/day |  |  | ≤once/shift |
|  |  |  |  | Severity (C) |  |  |  |  |  | First Aid Only |  |  | Medical Treatment |  |  | Lost Time |  |  |  |  | Partial Disability |  |  | Death or Permanent Disability |
|  |  |  |  | Likelihood (D) |  |  |  |  |  | Extremely Unlikely |  |  | Unlikely |  |  | Possible |  |  |  |  | Probable |  |  | Multiple |
|  |  |  | Before Additional Controls |  |  |  |  |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |  |  |  |
| Job Step/Task | Hazard | Control(s) | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D | Legal Exposure E |  | Risk* (AxBxCxD)+E | Control(s) Added to Reduce Risk |  |  | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D |  | Legal Exposure E | Risk* (AxBxCxD)+E | % Risk Reduction |
| Cleaning/ Nut bolt tightening /L.A changing | Stress | Hang gloves, safety belt, safety shoes, helmet, ladder, proper tools & tackle chain ton. | Y |  | 4 | 2 | 3 | 3 | 0 |  | 72 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fall down from height. |  | Y |  | 2 | 1 | 4 | 4 | 0 |  | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | L.A charge |  | Y |  | 5 | 1 | 5 | 2 | 0 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| During grounding led connection. | Voltage induction | Hand gloves safety belt, safety shoes, helmet, hot stick. | Y |  | 1 | 2 | 2 | 3 | 0 |  | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Measuring Insulation | LV shock | Hand gloves safety belt, safety shoes, helmet, grounding, | Y |  | 2 | 1 | 4 | 3 | 0 |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of electrical appliances | Electrocution or injury | All damaged electrical appliances shall be removed from service, use proper PPE | N |  | 2 | 4 | 2 | 3 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR LIGHTENING ARRESTOR |

(Contd.)

| Use of Metallic Tapes and Other Conductive Equipment | Electrocution or injury |  | Metallic tapes & conductive equipment shall avoid where applicable, use of proper PPE |  | N | 2 | 3 |  | 3 | 3 | 0 | 54 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  | 61 to80 |  |  |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  | Acceptable |  |  |  | Moderate |  |  |  |  |  | Substantial |  |  |  |  |  | Intolerable |  |  |  |

Each hazard should occupy one line in the risk table. That is, the risk from each hazard is to be assessed individually. A single activity must be entered three times in the table since there are three hazards associated with this activity.

Note: For legal exposure, the value should be either 5 for identified legal requirements or 0 for no such requirements could be identified.

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR SWITCH YARD (POTENTIAL TRANSFORMER) |

| Area/Facility Title: Switch yard (Potential Transformer) |  |  |  |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  | 1 |  |  | 2 |  | 3 |  |  |  | 4 |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Occupancy Or Use |  |  |  |  | ≤once/year |  |  | ≤once/month |  | ≤once/week |  |  |  | ≤once/day |  | ≤once/shift |
|  |  |  |  |  |  |  |  |  | Severity |  |  |  |  | First Aid Only |  |  | Medical Treatment |  | Lost Time |  |  |  | Partial Disability |  | Death or Permanent Disability |
|  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  | Very Unlikely |  |  | Unlikely |  | Possible |  |  |  | Probable |  | Multiple |
|  |  |  | Before Controls |  |  |  |  |  |  |  | After Initial Controls |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |
| Item or activity | Hazard (s) |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E | Risk* (AxBxC)+E | Initial Controls |  |  | Occupancy A | Severity B | Likelihood C |  | Legal Exposure E | Risk = (AxBxC)+E | Control(s) Added to Reduce Risk |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E |  | Risk* (AxBxC)+E | % Risk Reduction |
| Schedule & Emergency maintenance. | Fall from height |  | 3 | 5 | 4 | 0 | 60 | Hang gloves, safety belt, safety shoes, helmet, ladder. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | High Voltage shock |  | 3 | 5 | 4 | 0 | 60 | Proper grounding, hang gloves, safety belt, safety shoes, helmet, ladder |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Porcelain Blast |  | 1 | 5 | 5 | 0 | 25 | Safety shoes, helmet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bird or squirrel or snake |  | 4 | 2 | 4 | 0 | 32 | Inspection, horn, air gun, regular cleaning of grasses and bushes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overhead Lines | Contact with or near approach to a live high voltage |  | 5 | 4 | 4 | 0 | 80 | Use proper PPE, maintain safe distance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  |  |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  |  |  | Acceptable |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  | Intolerable |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR SWITCH YARD (POTENTIAL TRANSFORMER) |

| Job Title: Switch yard (Potential Transformer) Emergency or Schedule maintenance |  |  |  | Point Value → Parameter ↓ |  |  |  |  |  | 1 |  |  | 2 |  |  | 3 |  |  |  |  | 4 |  |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Frequency (B) |  |  |  |  |  | ≤once/year |  |  | ≤once/month |  |  | ≤once/week |  |  |  |  | ≤once/day |  |  | ≤once/shift |
|  |  |  |  | Severity (C) |  |  |  |  |  | First Aid Only |  |  | Medical Treatment |  |  | Lost Time |  |  |  |  | Partial Disability |  |  | Death or Permanent Disability |
|  |  |  |  | Likelihood (D) |  |  |  |  |  | Extremely Unlikely |  |  | Unlikely |  |  | Possible |  |  |  |  | Probable |  |  | Multiple |
|  |  |  | Before Additional Controls |  |  |  |  |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |  |  |  |
| Job Step/Task | Hazard | Control(s) | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D | Legal Exposure E |  | Risk* (AxBxCxD)+E | Control(s) Added to Reduce Risk |  |  | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D |  | Legal Exposure E | Risk* (AxBxCxD)+E | % Risk Reduction |
| Cleaning/ Nut bolt tightening /P.T changing | Stress | Hang gloves, safety belt, safety shoes, helmet, ladder, proper tools & tackle chain ton. | Y |  | 4 | 2 | 3 | 3 | 0 |  | 72 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fall down from height. |  | Y |  | 2 | 1 | 4 | 4 | 0 |  | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | P.T charge |  | Y |  | 5 | 1 | 5 | 2 | 0 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| During grounding led connection. | Voltage induction | Hand gloves safety belt, safety shoes, helmet, hot stick. | Y |  | 1 | 2 | 3 | 3 | 0 |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Measuring Insulation | LV shock | Hand gloves safety belt, safety shoes, helmet, grounding, | Y |  | 2 | 1 | 4 | 4 | 0 |  | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of electrical appliances | Electrocution or injury | All damaged electrical appliances shall be removed from service, use proper PPE | N |  | 2 | 4 | 2 | 3 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR SWITCH YARD (POTENTIAL TRANSFORMER) |

(Contd.)

| Use of Metallic Tapes and Other Conductive Equipment | Electrocution or injury |  | Metallic tapes & conductive equipment shall avoid where applicable, use of proper PPE |  | N | 2 | 3 |  | 3 | 3 | 0 | 54 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  | 61 to80 |  |  |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  | Acceptable |  |  |  | Moderate |  |  |  |  |  | Substantial |  |  |  |  |  | Intolerable |  |  |  |

Each hazard should occupy one line in the risk table. That is, the risk from each hazard is to be assessed individually. A single activity must be entered three times in the table since there are three hazards associated with this activity.

Note: For legal exposure, the value should be either 5 for identified legal requirements or 0 for no such requirements could be identified.

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR SWITCH YARD (POWER TRANSFORMER) |

| Area/Facility Title: Switch Yard (Power Transformer) |  |  |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  | 1 |  |  | 2 | 3 |  |  |  | 4 |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Occupancy Or Use |  |  |  | ≤once/year |  |  | ≤once/month | ≤once/week |  |  |  | ≤once/day |  | ≤once/shift |
|  |  |  |  |  |  |  |  | Severity |  |  |  | First Aid Only |  |  | Medical Treatment | Lost Time |  |  |  | Partial Disability |  | Death or Permanent Disability |
|  |  |  |  |  |  |  |  | Likelihood |  |  |  | Very Unlikely |  |  | Unlikely | Possible |  |  |  | Probable |  | Multiple |
|  |  | Before Controls |  |  |  |  |  |  | After Initial Controls |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |
| Item or activity | Hazard (s) | Occupancy A | Severity B | Likelihood C | Legal Exposure E | Risk* (AxBxC)+E | Initial Controls |  | Occupancy A | Severity B | Likelihood C |  | Legal Exposure E | Risk = (AxBxC)+E | Control(s) Added to Reduce Risk | Occupancy A | Severity B | Likelihood C | Legal Exposure E |  | Risk* (AxBxC)+E | % Risk Reduction |
| Schedule & emergency maintenance | HV shock | 2 | 5 | 3 | 0 | 30 | Proper isolation & grounding, hand gloves, safety shoes, safety belt. Helmet. Ladder. Safe distance from live parts. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | LV shock | 2 | 5 | 3 | 0 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fall down from height | 5 | 4 | 3 | 0 | 60 | Hand gloves, safety shoes, safety belt. Helmet. Ladder |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | HV shock | 2 | 5 | 3 | 0 | 30 | Proper isolation & grounding, hand gloves, safety shoes, safety belt. Helmet. Ladder. Safe distance from live parts. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transformer oil | High temperature due to short circuitcan can cause explosion | 4 | 4 | 4 | 0 | 64 | Cooling fan, protection system, oil schedule check. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR SWITCH YARD (POWER TRANSFORMER) |

(cond.)

| Transformer breather box | Presence of humidity in the air breather can cause explosion |  | 1 | 2 | 3 | 0 | 6 | Monitoring |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CT , PT,LA body | Burst & human injury |  | 1 | 2 | 3 | 0 | 6 | Regular checked any abnormality |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Switch Yard equipment | Electrocution |  | 4 | 5 | 4 | 0 | 80 | Follow safety sign, Use appropriate PPE, proper earthing. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overhead Lines | Contact with or near approach to a live high voltage |  | 5 | 4 | 4 | 0 | 80 | Use proper PPE, maintain safe distance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  |  |  | 21 to40 |  |  | 41 to60 |  |  |  |  |  |  | 61 to80 |  |  |  | 81 or greater |  |  |
|  |  | Negligible |  |  |  | Acceptable |  |  | Moderate |  |  |  |  |  |  | Substantial |  |  |  | Intolerable |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR SWITCH YARD (POWER TRANSFORMER) |

| Job Title: Switch Yard (Power Transformer) Power Transformer Schedule & Emergency Maintenance |  |  |  | Point Value → Parameter ↓ |  |  |  |  |  | 1 |  |  | 2 |  |  | 3 |  |  |  |  | 4 |  |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Frequency (B) |  |  |  |  |  | ≤once/year |  |  | ≤once/month |  |  | ≤once/week |  |  |  |  | ≤once/day |  |  | ≤once/shift |
|  |  |  |  | Severity (C) |  |  |  |  |  | First Aid Only |  |  | Medical Treatment |  |  | Lost Time |  |  |  |  | Partial Disability |  |  | Death or Permanent Disability |
|  |  |  |  | Likelihood (D) |  |  |  |  |  | Extremely Unlikely |  |  | Unlikely |  |  | Possible |  |  |  |  | Probable |  |  | Multiple |
|  |  |  | Before Additional Controls |  |  |  |  |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |  |  |  |
| Job Step/Task | Hazard | Control(s) | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D | Legal Exposure E |  | Risk* (AxBxCxD)+E | Control(s) Added to Reduce Risk |  |  | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D |  | Legal Exposure E | Risk* (AxBxCxD)+E | % Risk Reduction |
| Cleaning/ Tightening | Spillage | Proper isolation & grounding, hand gloves, safety shoes, safety belt. Helmet. Ladder | Y |  | 4 | 2 | 3 | 3 | 0 |  | 72 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Electrical shock |  | Y |  | 2 | 2 | 3 | 3 | 0 |  | 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Short circuit |  | Y |  | 4 | 2 | 2 | 3 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Insulation measurement | Electrical shock | Hand gloves, safety shoes, safety belt. Helmet. Proper grounding. | Y |  | 2 | 2 | 3 | 3 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Short circuit |  | N |  | 2 | 2 | 2 | 4 | 0 |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Testing (O.C, S.C, Tan-delta, SFRA, Winding resistance etc. ) | Electrical shock | Hand gloves, safety shoes, safety belt. Helmet. Proper grounding | Y |  | 2 | 2 | 2 | 3 | 0 |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Short circuit |  | N |  | 2 | 2 | 2 | 4 | 0 |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspection | Suffocation | Proper ventilation | y |  | 1 | 1 | 5 | 5 | 0 |  | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of electrical appliances | Electrocution or injury | All damaged electrical appliances shall be removed from service, use proper PPE | N |  | 2 | 4 | 2 | 3 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR SWITCH YARD (POWER TRANSFORMER) |

(Contd.)

| Use of Metallic Tapes and Other Conductive Equipment | Electrocution or injury |  | Metallic tapes & conductive equipment shall avoid where applicable, use of proper PPE |  | N | 2 | 3 |  | 3 | 3 | 0 | 54 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  | 61 to80 |  |  |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  | Acceptable |  |  |  | Moderate |  |  |  |  |  | Substantial |  |  |  |  |  | Intolerable |  |  |  |

Each hazard should occupy one line in the risk table. That is, the risk from each hazard is to be assessed individually. A single activity must be entered three times in the table since there are three hazards associated with this activity.

Note: For legal exposure, the value should be either 5 for identified legal requirements or 0 for no such requirements could be identified.

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR RELAY PANEL |

| Area/Facility Title: Relay Panel |  |  |  |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  | 1 |  |  | 2 |  | 3 |  |  |  | 4 |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Occupancy Or Use |  |  |  |  | ≤once/year |  |  | ≤once/month |  | ≤once/week |  |  |  | ≤once/day |  | ≤once/shift |
|  |  |  |  |  |  |  |  |  | Severity |  |  |  |  | First Aid Only |  |  | Medical Treatment |  | Lost Time |  |  |  | Partial Disability |  | Death or Permanent Disability |
|  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  | Very Unlikely |  |  | Unlikely |  | Possible |  |  |  | Probable |  | Multiple |
|  |  |  | Before Controls |  |  |  |  |  |  |  | After Initial Controls |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |
| Item or activity | Hazard (s) |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E | Risk* (AxBxC)+E | Initial Controls |  |  | Occupancy A | Severity B | Likelihood C |  | Legal Exposure E | Risk = (AxBxC)+E | Control(s) Added to Reduce Risk |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E |  | Risk* (AxBxC)+E | % Risk Reduction |
| Maintenance, testing & troubleshooting | Compact area |  | 1 | 3 | 3 | 0 | 9 | Proper isolation & grounding, hand gloves, safety shoes, Safety belt. helmet. Safe distance from live parts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | LV AC shock |  | 1 | 2 | 5 | 0 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | DC shock |  | 1 | 4 | 3 | 0 | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | HV open CT shock |  | 1 | 5 | 3 | 0 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Electrical heater |  | 1 | 1 | 3 | 0 | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Body not earthed properly |  | 5 | 2 | 1 | 0 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overhead Lines | Contact with or near approach to a live high voltage |  | 5 | 4 | 4 | 0 | 80 | Use proper PPE, maintain safe distance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  |  |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  |  |  | Acceptable |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  | Intolerable |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR RELAY PANEL |

| Job Title: Relay Panel Maintenance/ Test/ Troubleshooting |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  |  |  | 1 |  |  | 2 |  |  |  | 3 |  |  |  |  | 4 |  |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Frequency (B) |  |  |  |  |  |  | ≤once/year |  |  | ≤once/month |  |  |  | ≤once/week |  |  |  |  | ≤once/day |  |  | ≤once/shift |
|  |  |  |  |  |  | Severity (C) |  |  |  |  |  |  | First Aid Only |  |  | Medical Treatment |  |  |  | Lost Time |  |  |  |  | Partial Disability |  |  | Death or Permanent Disability |
|  |  |  |  |  |  | Likelihood (D) |  |  |  |  |  |  | Extremely Unlikely |  |  | Unlikely |  |  |  | Possible |  |  |  |  | Probable |  |  | Multiple |
|  |  |  |  |  | Before Additional Controls |  |  |  |  |  |  |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |  |  |  |
| Job Step/Task | Hazard |  | Control(s) |  | Stressors Y/N |  | # of People A | Frequency B |  | Severity C | Likelihood D | Legal Exposure E |  | Risk* (AxBxCxD)+E | Control(s) Added to Reduce Risk |  |  |  | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D |  | Legal Exposure E | Risk* (AxBxCxD)+E | % Risk Reduction |
| Cleaning/ wiring/ continuity test/ relay functionality test. | HV open ct shock |  | Proper isolation & grounding, hand gloves, safety shoes, safety belt. Helmet. Safe distance from live parts |  | N |  | 2 | 1 |  | 5 | 2 | 0 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | LV ac shock |  |  |  | N |  | 2 | 1 |  | 2 | 5 | 0 |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dc shock |  |  |  | N |  | 2 | 1 |  | 4 | 2 | 0 |  | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Burning from heater |  |  |  | N |  | 2 | 1 |  | 1 | 2 | 0 |  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pinching |  |  |  | N |  | 2 | 1 |  | 1 | 2 | 0 |  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of electrical appliances | Electrocution or injury |  | All damaged electrical appliances shall be removed from service, use proper PPE |  | N |  | 2 | 4 |  | 2 | 3 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of Metallic Tapes and Other Conductive Equipment | Electrocution or injury |  | Metallic tapes & conductive equipment shall avoid where applicable, use of proper PPE |  | N |  | 2 | 3 |  | 3 | 3 | 0 |  | 54 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  | 21 to40 |  |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  |  |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  | Acceptable |  |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  |  |  |  |  | Intolerable |  |  |  |

Each hazard should occupy one line in the risk table. That is, the risk from each hazard is to be assessed individually. A single activity must be entered three times in the table since there are three hazards associated with this activity.

Note: For legal exposure, the value should be either 5 for identified legal requirements or 0 for no such requirements could be identified.

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR SF6 GAS FILLING |

| Area/Facility Title: Switch yard (SF6 Gas Filling in C.B) |  |  |  |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  | 1 |  |  | 2 |  | 3 |  |  |  | 4 |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Occupancy Or Use |  |  |  |  | ≤once/year |  |  | ≤once/month |  | ≤once/week |  |  |  | ≤once/day |  | ≤once/shift |
|  |  |  |  |  |  |  |  |  | Severity |  |  |  |  | First Aid Only |  |  | Medical Treatment |  | Lost Time |  |  |  | Partial Disability |  | Death or Permanent Disability |
|  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  | Very Unlikely |  |  | Unlikely |  | Possible |  |  |  | Probable |  | Multiple |
|  |  |  | Before Controls |  |  |  |  |  |  |  | After Initial Controls |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |
| Item or activity | Hazard (s) |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E | Risk* (AxBxC)+E | Initial Controls |  |  | Occupancy A | Severity B | Likelihood C |  | Legal Exposure E | Risk = (AxBxC)+E | Control(s) Added to Reduce Risk |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E |  | Risk* (AxBxC)+E | % Risk Reduction |
| Gas filling | High pressure gas |  | 3 | 3 | 4 | 0 | 36 | Hand gloves, musk, safety shoes, safety belt, helmet. Maintain proper instructions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  |  |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  |  |  | Acceptable |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  | Intolerable |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR SF6 GAS FILLING |

| Job Title: Switch yard (SF6 Gas Filling in C.B) |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  |  |  | 1 |  |  | 2 |  |  |  | 3 |  |  |  |  | 4 |  |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Frequency (B) |  |  |  |  |  |  | ≤once/year |  |  | ≤once/month |  |  |  | ≤once/week |  |  |  |  | ≤once/day |  |  | ≤once/shift |
|  |  |  |  |  |  | Severity (C) |  |  |  |  |  |  | First Aid Only |  |  | Medical Treatment |  |  |  | Lost Time |  |  |  |  | Partial Disability |  |  | Death or Permanent Disability |
|  |  |  |  |  |  | Likelihood (D) |  |  |  |  |  |  | Extremely Unlikely |  |  | Unlikely |  |  |  | Possible |  |  |  |  | Probable |  |  | Multiple |
|  |  |  |  |  | Before Additional Controls |  |  |  |  |  |  |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |  |  |  |
| Job step/task | Hazard |  | Control(s) |  | Stressors Y/N |  | # of People A | Frequency B |  | Severity C | Likelihood D | Legal Exposure E |  | Risk* (AxBxCxD)+E | Control(s) Added to Reduce Risk |  |  |  | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D |  | Legal Exposure E | Risk* (AxBxCxD)+E | % Risk Reduction |
| Rolling gas cylinder from trolley | Cylinder fall down |  | Hand gloves, musk, safety shoes, safety belt, helmet, careful carrying |  | Y |  | 3 | 3 |  | 3 | 2 | 0 |  | 54 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gas filling | Gas leakage |  |  |  | N |  | 2 | 2 |  | 4 | 2 | 0 |  | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Leakage of gas pipe |  |  |  | Y |  | 2 | 2 |  | 3 | 2 | 0 |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Without regulator filling | Over charge |  |  |  | Y |  | 3 | 3 |  | 4 | 1 | 0 |  | 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| During opening & tithing nozzle. | Man fall down from height. |  | Hand gloves, safety shoes, safety belt, helmet. |  | Y |  | 2 | 2 |  | 4 | 3 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  | 21 to40 |  |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  |  |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  | Acceptable |  |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  |  |  |  |  | Intolerable |  |  |  |

Each hazard should occupy one line in the risk table. That is, the risk from each hazard is to be assessed individually. A single activity must be entered three times in the table since there are three hazards associated with this activity.

Note: For legal exposure, the value should be either 5 for identified legal requirements or 0 for no such requirements could be identified.

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR TRANSMISSION LINE |

| Area/Facility Title: Transmission Line |  |  |  |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  | 1 |  |  | 2 |  | 3 |  |  |  | 4 |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Occupancy Or Use |  |  |  |  | ≤once/year |  |  | ≤once/month |  | ≤once/week |  |  |  | ≤once/day |  | ≤once/shift |
|  |  |  |  |  |  |  |  |  | Severity |  |  |  |  | First Aid Only |  |  | Medical Treatment |  | Lost Time |  |  |  | Partial Disability |  | Death or Permanent Disability |
|  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  | Very Unlikely |  |  | Unlikely |  | Possible |  |  |  | Probable |  | Multiple |
|  |  |  | Before Controls |  |  |  |  |  |  |  | After Initial Controls |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |
| Item or activity | Hazard (s) |  | Occupancy a | Severity b | Likelihood c | Legal exposure E | Risk* (axbxc)+e | Initial controls |  |  | Occupancy A | Severity B | Likelihood C |  | Legal Exposure E | Risk = (AxBxC)+E | Control(s) Added to Reduce Risk |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E |  | Risk* (AxBxC)+E | % Risk Reduction |
| Schedule and emergency maintenance | Hv shock |  | 2 | 5 | 4 | 0 | 40 | Proper isolation & grounding, hand gloves, safety shoes, safety belt. Helmet. Proper use of ladder. Safe distance from live parts. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fall down from height |  | 1 | 5 | 4 | 0 | 20 | Safety belt. Helmet. Proper use of ladder |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overhead Lines | Contact with or near approach to a live high voltage |  | 5 | 4 | 4 | 0 | 80 | Use proper PPE, maintain safe distance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  |  |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  |  |  | Acceptable |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  | Intolerable |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR TRANSMISSION LINE |

| Job Title: Transmission Line Schedule & Emergency Maintenance |  |  |  | Point Value → Parameter ↓ |  |  |  |  |  | 1 |  |  | 2 |  |  | 3 |  |  |  |  | 4 |  |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Frequency (B) |  |  |  |  |  | ≤once/year |  |  | ≤once/month |  |  | ≤once/week |  |  |  |  | ≤once/day |  |  | ≤once/shift |
|  |  |  |  | Severity (C) |  |  |  |  |  | First Aid Only |  |  | Medical Treatment |  |  | Lost Time |  |  |  |  | Partial Disability |  |  | Death or Permanent Disability |
|  |  |  |  | Likelihood (D) |  |  |  |  |  | Extremely Unlikely |  |  | Unlikely |  |  | Possible |  |  |  |  | Probable |  |  | Multiple |
|  |  |  | Before Additional Controls |  |  |  |  |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |  |  |  |
| Job Step/Task | Hazard | Control(s) | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D | Legal Exposure E |  | Risk* (AxBxCxD)+E | Control(s) Added to Reduce Risk |  |  | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D |  | Legal Exposure E | Risk* (AxBxCxD)+E | % Risk Reduction |
| Cleaning/ tightening/ nut bolt changing, | Stressors | Proper isolation & grounding, hand gloves, safety shoes, safety belt. Helmet. Ladder | Y |  | 2 | 2 | 3 | 3 | 0 |  | 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fall down from height |  | Y |  | 2 | 1 | 5 | 4 | 0 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Changing disc insulator. Tower angle replacement | Stressors |  | Y |  | 2 | 2 | 2 | 3 | 0 |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fall down from height |  | N |  | 2 | 2 | 3 | 4 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Row or tree trimming. | Stressors | Proper isolation & grounding, hand gloves, safety shoes, safety belt. Helmet. Ladder, saw, rough, hot stick | Y |  | 4 | 2 | 4 | 2 | 0 |  | 64 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fall down from height |  | Y |  | 4 | 2 | 4 | 2 | 0 |  | 64 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Hv shock |  | Y |  | 2 | 2 | 5 | 3 | 0 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of electrical appliances | Electrocution or injury | All damaged electrical appliances shall be removed from service, use proper PPE | N |  | 2 | 4 | 2 | 3 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR TRANSMISSION LINE |

(Contd.)

| Use of Metallic Tapes and Other Conductive Equipment | Electrocution or injury |  | Metallic tapes & conductive equipment shall avoid where applicable, use of proper PPE |  | N | 2 | 3 |  | 3 | 3 | 0 | 54 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  | 61 to80 |  |  |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  | Acceptable |  |  |  | Moderate |  |  |  |  |  | Substantial |  |  |  |  |  | Intolerable |  |  |  |

Each hazard should occupy one line in the risk table. That is, the risk from each hazard is to be assessed individually. A single activity must be entered three times in the table since there are three hazards associated with this activity.

Note: For legal exposure, the value should be either 5 for identified legal requirements or 0 for no such requirements could be identified.

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR FIRE FIGHTING SYSTEM |

| Area/Facility Title: Fire Fighting System |  |  |  |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  | 1 |  |  | 2 |  | 3 |  |  |  | 4 |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Occupancy Or Use |  |  |  |  | ≤once/year |  |  | ≤once/month |  | ≤once/week |  |  |  | ≤once/day |  | ≤once/shift |
|  |  |  |  |  |  |  |  |  | Severity |  |  |  |  | First Aid Only |  |  | Medical Treatment |  | Lost Time |  |  |  | Partial Disability |  | Death or Permanent Disability |
|  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  | Very Unlikely |  |  | Unlikely |  | Possible |  |  |  | Probable |  | Multiple |
|  |  |  | Before Controls |  |  |  |  |  |  |  | After Initial Controls |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |
| Item or activity | Hazard (s) |  | Occupancy a | Severity b | Likelihood c | Legal exposure E | Risk* (axbxc)+e | Initial controls |  |  | Occupancy A | Severity B | Likelihood C |  | Legal Exposure E | Risk = (AxBxC)+E | Control(s) Added to Reduce Risk |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E |  | Risk* (AxBxC)+E | % Risk Reduction |
| Fire Fighting system | Fire |  | 2 | 2 | 2 | 0 | 8 | PPE & proper safety instruction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Portable ABC Dry Powder | Pressurized Cylinder |  | 1 | 1 | 2 | 0 | 2 | Protection as per Safety instruction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Portable CO2 Extinguisher | Pressurized Cylinder |  | 1 | 1 | 2 | 0 | 2 | Protection as per Safety instruction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DCP Extinguisher | Pressurized Cylinder |  | 1 | 1 | 2 | 0 | 2 | Protection as per Safety instruction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Portable Dry Powder | Pressurized Cylinder |  | 1 | 1 | 2 | 0 | 2 | Protection as per Safety instruction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  |  |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  |  |  | Acceptable |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  | Intolerable |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR FIRE FIGHTING SYSTEM |

| Job Title: Fire fighting system |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  |  |  | 1 |  |  | 2 |  |  |  | 3 |  |  |  |  | 4 |  |  | 5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Frequency (B) |  |  |  |  |  |  | ≤once/year |  |  | ≤once/month |  |  |  | ≤once/week |  |  |  |  | ≤once/day |  |  | ≤once/shift |  |
|  |  |  |  |  |  | Severity (C) |  |  |  |  |  |  | First Aid Only |  |  | Medical Treatment |  |  |  | Lost Time |  |  |  |  | Partial Disability |  |  | Death or Permanent Disability |  |
|  |  |  |  |  |  | Likelihood (D) |  |  |  |  |  |  | Extremely Unlikely |  |  | Unlikely |  |  |  | Possible |  |  |  |  | Probable |  |  | Multiple |  |
|  |  |  |  |  | Before Additional Controls |  |  |  |  |  |  |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |  |  |  |  |
| Job Step/Task | Hazard |  | Control(s) |  | Stressors Y/N |  | # of People A | Frequency B |  | Severity C | Likelihood D | Legal Exposure E |  | Risk* (AxBxCxD)+E | Control(s) Added to Reduce Risk |  |  |  | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D |  | Legal Exposure E | Risk* (AxBxCxD)+E | % Risk Reduction |
| Use of fire extinguisher | High pressure gas |  | Follow instructions carefully |  | Y |  | 3 | 1 |  | 2 | 2 | 0 |  | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  | 21 to40 |  |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  |  |  |  |  | 81 or greater |  |  |  |  |
|  |  | Negligible |  | Acceptable |  |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  |  |  |  |  | Intolerable |  |  |  |  |

Each hazard should occupy one line in the risk table. That is, the risk from each hazard is to be assessed individually. A single activity must be entered three times in the table since there are three hazards associated with this activity.

Note: For legal exposure, the value should be either 5 for identified legal requirements or 0 for no such requirements could be identified.

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR OFFICE |

| Area/Facility Title: Office |  |  |  |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  | 1 |  |  | 2 |  | 3 |  |  |  | 4 |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Occupancy Or Use |  |  |  |  | ≤once/year |  |  | ≤once/month |  | ≤once/week |  |  |  | ≤once/day |  | ≤once/shift |
|  |  |  |  |  |  |  |  |  | Severity |  |  |  |  | First Aid Only |  |  | Medical Treatment |  | Lost Time |  |  |  | Partial Disability |  | Death or Permanent Disability |
|  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  | Very Unlikely |  |  | Unlikely |  | Possible |  |  |  | Probable |  | Multiple |
|  |  |  | Before Controls |  |  |  |  |  |  |  | After Initial Controls |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |
| Item or activity | Hazard (s) |  | Occupancy a | Severity b | Likelihood c | Legal exposure E | Risk* (axbxc)+e | Initial controls |  |  | Occupancy A | Severity B | Likelihood C |  | Legal Exposure E | Risk = (AxBxC)+E | Control(s) Added to Reduce Risk |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E |  | Risk* (AxBxC)+E | % Risk Reduction |
| Office | Posture, Eyestrain |  | 5 | 1 | 4 | 0 | 20 | Protection as per Safety instruction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Computer, Printer | Posture, Eyestrain |  | 5 | 1 | 4 | 0 | 20 | Protection as per Safety instruction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| computer | electricity, radiation |  | 5 | 1 | 4 | 0 | 20 | Protection as per Safety instruction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Toilets | Human Waste |  | 2 | 2 | 3 | 0 | 12 | Clean & disposal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Electric heater | Fire, Electric shock |  | 5 | 1 | 4 | 0 | 20 | Maintain safety cautions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  |  |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  |  |  | Acceptable |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  | Intolerable |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR OFFICE |

| Job Title: Office |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  |  |  | 1 |  |  | 2 |  |  |  | 3 |  |  |  |  | 4 |  |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Frequency (B) |  |  |  |  |  |  | ≤once/year |  |  | ≤once/month |  |  |  | ≤once/week |  |  |  |  | ≤once/day |  |  | ≤once/shift |
|  |  |  |  |  |  | Severity (C) |  |  |  |  |  |  | First Aid Only |  |  | Medical Treatment |  |  |  | Lost Time |  |  |  |  | Partial Disability |  |  | Death or Permanent Disability |
|  |  |  |  |  |  | Likelihood (D) |  |  |  |  |  |  | Extremely Unlikely |  |  | Unlikely |  |  |  | Possible |  |  |  |  | Probable |  |  | Multiple |
|  |  |  |  |  | Before Additional Controls |  |  |  |  |  |  |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |  |  |  |
| Job Step/Task | Hazard |  | Control(s) |  | Stressors Y/N |  | # of People A | Frequency B |  | Severity C | Likelihood D | Legal Exposure E |  | Risk* (AxBxCxD)+E | Control(s) Added to Reduce Risk |  |  |  | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D |  | Legal Exposure E | Risk* (AxBxCxD)+E | % Risk Reduction |
| Improper sitting | Back pain |  | Follow ergonomics |  | Y |  | 4 | 5 |  | 1 | 3 | 0 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of computer | Eye strain, Headache |  | Follow ergonomics |  | Y |  | 4 | 5 |  | 1 | 3 | 0 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of electric heater | Fire, Electric shock |  | Maintain safety cautions |  | Y |  | 1 | 5 |  | 1 | 3 | 0 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of other electrical equipment | Fire, Electric shock |  | Maintain safety cautions |  | Y |  | 1 | 5 |  | 1 | 3 | 0 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Use of other electrical appliances | Electrocution or injury |  | All damaged electrical appliances shall be removed from service, use proper PPE |  | N |  | 2 | 4 |  | 2 | 3 | 0 |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  | 21 to40 |  |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  |  |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  | Acceptable |  |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  |  |  |  |  | Intolerable |  |  |  |

Each hazard should occupy one line in the risk table. That is, the risk from each hazard is to be assessed individually. A single activity must be entered three times in the table since there are three hazards associated with this activity.

Note: For legal exposure, the value should be either 5 for identified legal requirements or 0 for no such requirements could be identified.

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | FACILITY RISK ASSESSMENT FOR STORAGE |

| Area/Facility Title: Storage |  |  |  |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  | 1 |  |  | 2 |  | 3 |  |  |  | 4 |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Occupancy Or Use |  |  |  |  | ≤once/year |  |  | ≤once/month |  | ≤once/week |  |  |  | ≤once/day |  | ≤once/shift |
|  |  |  |  |  |  |  |  |  | Severity |  |  |  |  | First Aid Only |  |  | Medical Treatment |  | Lost Time |  |  |  | Partial Disability |  | Death or Permanent Disability |
|  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  | Very Unlikely |  |  | Unlikely |  | Possible |  |  |  | Probable |  | Multiple |
|  |  |  | Before Controls |  |  |  |  |  |  |  | After Initial Controls |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |
| Item or activity | Hazard (s) |  | Occupancy a | Severity b | Likelihood c | Legal exposure E | Risk* (axbxc)+e | Initial controls |  |  | Occupancy A | Severity B | Likelihood C |  | Legal Exposure E | Risk = (AxBxC)+E | Control(s) Added to Reduce Risk |  | Occupancy A | Severity B | Likelihood C | Legal Exposure E |  | Risk* (AxBxC)+E | % Risk Reduction |
| Hazardous material storage | Trimming & fall |  | 4 | 1 | 3 | 0 | 12 | Use of PPE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lube Oil Storage | Oil Spill and leakage |  | 3 | 1 | 4 | 0 | 12 | Use of PPE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Waste materials storage | Trimming & fall |  | 4 | 1 | 3 | 0 | 12 | Use of PPE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  |  |  | 21 to40 |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  |  |  | Acceptable |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  | Intolerable |  |  |  |

|  | POWER GRID COMPANY OF BANGLADESH LTD. |
| --- | --- |
|  | JOB RISK ASSESSMENT FOR STORAGE |

| Job Title: Storage |  |  |  |  |  | Point Value → Parameter ↓ |  |  |  |  |  |  | 1 |  |  | 2 |  |  |  | 3 |  |  |  |  | 4 |  |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Frequency (B) |  |  |  |  |  |  | ≤once/year |  |  | ≤once/month |  |  |  | ≤once/week |  |  |  |  | ≤once/day |  |  | ≤once/shift |
|  |  |  |  |  |  | Severity (C) |  |  |  |  |  |  | First Aid Only |  |  | Medical Treatment |  |  |  | Lost Time |  |  |  |  | Partial Disability |  |  | Death or Permanent Disability |
|  |  |  |  |  |  | Likelihood (D) |  |  |  |  |  |  | Extremely Unlikely |  |  | Unlikely |  |  |  | Possible |  |  |  |  | Probable |  |  | Multiple |
|  |  |  |  |  | Before Additional Controls |  |  |  |  |  |  |  |  |  |  |  |  |  | After Additional Controls |  |  |  |  |  |  |  |  |  |
| Job Step/Task | Hazard |  | Control(s) |  | Stressors Y/N |  | # of People A | Frequency B |  | Severity C | Likelihood D | Legal Exposure E |  | Risk* (AxBxCxD)+E | Control(s) Added to Reduce Risk |  |  |  | Stressors Y/N |  | # of People A | Frequency B | Severity C | Likelihood D |  | Legal Exposure E | Risk* (AxBxCxD)+E | % Risk Reduction |
| Walking | Tripping |  | Sufficient light, Maintain safety cautions |  | N |  | 2 | 4 |  | 1 | 4 | 0 |  | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Picking any material | Falling |  | Maintain safety cautions, Use of PPE |  | N |  | 2 | 4 |  | 1 | 4 | 0 |  | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Further Description of Controls Added to Reduce Risk: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Risk |  | 0 to20 |  | 21 to40 |  |  |  |  | 41 to60 |  |  |  |  |  |  |  | 61 to80 |  |  |  |  |  |  |  | 81 or greater |  |  |  |
|  |  | Negligible |  | Acceptable |  |  |  |  | Moderate |  |  |  |  |  |  |  | Substantial |  |  |  |  |  |  |  | Intolerable |  |  |  |

Each hazard should occupy one line in the risk table. That is, the risk from each hazard is to be assessed individually. A single activity must be entered three times in the table since there are three hazards associated with this activity.

Note: For legal exposure, the value should be either 5 for identified legal requirements or 0 for no such requirements could be identified.